WHICH MACHINE AM I RUNNING ON?

by Dilwyn Jones and Marcel Kilgus

(updated 24/01/13 for SMSQmulator Machine Type)

An interesting problem which faces QL programmers now is testing to see which of the multitude of QL-Compatibles the program is actually running on. To facilitate this, a largely undocumented system variable has been added and SBASIC for example has inbuilt BASIC keywords to test and return values associated with this. In general, software would of course run on any machine type, but it is possible that assembler programmers may wish to identify the machine or processor type to build time-critical loops, or to take advantage of facilities available on more recent machines by introducing two routines, one to take advantage of the new facilites to speed up or otherwise improve the program, the other a more general set of code to work on all machines.

The new system variable sys_ptyp at offset $A1 (decimal 161) contains a byte holding values which identify the type of processor and if any floating point unit is available.

The top 4 bits (high nibble, or bits 4-7) identify the processor type:

HEX	DECIMAL	PROCESSOR TYPE

0x	0+	68000 or 68008

1x	16+	68010

2x	32+	68020

3x	48+	68030

4x	64+	68040

6x	96+	68060

The bottom 4 bits (lower nibble, or bits 0-3) contains information about any floating point unit available. It takes the form:

HEX	DECIMAL	FPU DETAILS

0	0	No FPU available

1	1	Internal MMU

2	2	68851 MMU

4	4	internal FPU

8	8	68881 or 68882 FPU

The new system variable at $A7 (decimal 167) contains a byte holding information about the machine type. Bit 0 indicates if a Hermes or Blitter chip is installed (depending on machine), bits 1 to 4 indicate the machine or emulator type, while bits 5 to 7 indicate the display type as shown in Marcel's list below.

sys_mtyp equ    $00a7   byte         Machine type / display type

HEX   DECIMAL

sys.mtyp equ      $1e   30           machine ID in these bits

; bit 0

sys.blit  equ      +1                Blitter fitted (Atari)

sys.herm  equ      +1                Hermes fitted (QL)

;bits 1 to 4...

sys.mst   equ     $00   0            ordinary ST or QL

sys.mstr  equ     $02   2            Mega ST or ST with RTC

sys.msta  equ     $04   4            Stacy

sys.mste  equ     $06   6            ordinary STE

sys.mmste equ     $08   8            Mega STE

sys.mgold equ     $0a   10           Gold Card

sys.msgld equ     $0c   12           Super Gold Card

sys.mfal  equ     $10   16           Falcon

sys.mq40  equ     $11   17           Q40

sys.msms  equ     $14   20           SMSQmulator

sys.mtt   equ     $18   24           TT 030

sys.mqxl  equ     $1c   28           QXL

sys.mqpc  equ     $1e   30           QPC

sys.mqlay equ     $1f   31           QLay emulator

;bits 5 to 7...

sys.mfut  equ    %00000000 (dec. +0)     Standard QL or Futura type of display

sys.mmon  equ    %00100000 (dec. +32)    Monochrome monitor

sys.mext  equ    %01000000 (dec. +64)    Atari Extended 4 Emulator

sys.mvme  equ    %10000000 (dec. +128)   Atari QVME emulator

sys.mqlc  equ    %11000000 (dec. +192)   QL mode LCD

sys.mvga  equ    %11000000 (dec. +192)   VGA

sys.maur  equ    %10100000 (dec. +160)   Aurora

To locate these system variables from assembler, ensure you call sms.info (trap #0, d0=$00, system variables address returned in a0) then add the offsets mentioned to the base address and analyse the values at those locations. From BASIC, use any of the commonly available toolkit extensions to locate the system variables, for example, on Minerva or SMSQE yoy can use the VER$(-2) function to return the base address of the system variables.

EXAMPLES

This little BASIC program will list the information it can extract, somewhat along the lines of the equivalent SBASIC extensions MACHINE, PROCESSOR and DISP_TYPE in SMSQ/E.

100 sv = 163840 : REMark fixed location on original QL

110 t$ = VER$   : REMark check version of BASIC

120 IF t$ = 'JSL1' OR t$ = 'HBA' THEN

130   REMark Minerva or SBASIC, supports ver$(-2)

140   sv = VER$(-2)

150 END IF

160 processor_type = PEEK(sv+161)

170 REMark separate processor and FPU type

180 fpu_type = processor_type && 15

190 processor_type = processor_type DIV 16

200 machine_type = PEEK(sv+167)

210 REMark mask out machine_type into the machine and display components

220 display_type = machine_type DIV 32

230 machine_type = machine_type && 31

240 PRINT"Processor Type:";

250 SELect ON processor_type

260   =0 : PRINT"68000 or 68008"

270   =1 : PRINT"68010 (possibly QPC)"

280   =2 : PRINT"68020"

290   =3 : PRINT"68030"

300   =4 : PRINT"68040"

400   =5 : PRINT"Someone must have made a 68050 unit!"

410   =6 : PRINT"68060"

420   =REMAINDER:PRINT"Unknown"

430 END SELect

440 IF fpu_type > 0 THEN PRINT"FPU available"

450 PRINT"Display type:";

460 SELect ON display_type

470   =0 : PRINT"QL style display"

480   =1 : PRINT"Monochrome display"

490   =2 : PRINT"Atari Extended Mode 4"

500   =4 : PRINT"Atari QVME display"

510   =5 : PRINT"Aurora display"

520   =6 : PRINT"QL Mode LCD"

530   =7 : PRINT"VGA displays"

540   =REMAINDER : PRINT"Unknown"

550 END SELect

560 PRINT"Machine Type:";

570 SELect ON machine_type

580   =0,1  : PRINT"Unknown, probably a standard QL"

580   =2,3  : PRINT"Mega ST or ST with RTC"

590   =4,5  : PRINT"Stacy"

600   =6,7  : PRINT"Atari STE"

610   =8,9  : PRINT"Atari Mega STE"

620   =10,11: PRINT"Gold Card"

630   =12,13: PRINT"Super Gold Card"

640   =16,17: PRINT"Falcon"

650   =17,18: PRINT"Q40"

655   =20,21: PRINT"SMSQmulator"

660   =24,25: PRINT"Atari TT 030"

670   =28,29: PRINT"QXL"

680   =30,31: PRINT"QPC"

690   =REMAINDER : PRINT"Unknown"

700 END SELect

710 IF machine_type && 1 : PRINT"Hermes or Blitter installed"

Or if you prefer a set of functions to return the hardware details, try these:

1000 DEFine FuNction S_V

1010   LOCal t$

1020   t$ = VER$

1030   IF t$ = 'JSL1' OR t$ = 'HBA' THEN

1040     RETurn VER$(-2)

1050   ELSE

1060     RETurn 163840 : REMark QDOS QL fixed location

1070   END IF

1080 END DEFine S_V

1090 :

1100 DEFine Function MACHINE_TYPE

1110   LOCal mt

1120   mt = PEEK(S_V+167)

1130   RETurn (mt && 31)

1140 END DEFine MACHINE_TYPE

1150 :

1160 DEFine FuNction DISPLAY_TYPE

1170   LOCal mt

1180   mt = PEEK(S_V+167) && (128+64+32)

1190   RETurn (mt DIV 32)

1200 END DEFine DISPLAY_TYPE

1210 :

1220 DEFine FuNction PROCESSOR_TYPE

1230   LOCal pt

1240   pt = PEEK(S_V+161)

1250   RETurn (pt DIV 16)

1260 END DEFine PROCESSOR_TYPE

1270 :

1280 DEFine FuNction DISPLAY_TYPE

1290   LOCal pt

1300   pt = PEEK(S_V+161)

1310   RETurn (pt && 15)

1320 END DEFine DISPLAY_TYPE

1330 :

1340 DEFine FuNction HERMES_OR_BLITTER

1350   LOCal mt

1360   mt = PEEK(S_V+167)

1370   RETurn (mt && 1)

1380 END DEFine HERMES_OR_BLITTER